Register Number:

Date & Session :

ST.JOSEPH’S UNIVERSITY, BANGALORE-27

B.Sc. ELECTRONICS – III SEMESTER

SEMESTER EXAMINATION- OCTOBER 2023

(Examination conducted in November/December 2023)

EL 322- DIGITAL ELECTRONICS AND VERILOG HDL

(For current batch students only)

Time: 2 hrs                                                                                                             Max marks: 60

This question paper has TWO printed pages and THREE parts.

PART – A

ANSWER ALL THE FOLLOWING						       1x10=10

- TTL device consumes substantially _____ power than an equivalent CMOS device.
a) Less 		b) More		c) Equal		d) Very High

- 3 bit full adder contains ________
a) 3 combinational inputs				b) 4 combinational inputs
c) 6 combinational inputs				d) 8 combinational inputs

- Which combinational circuit is renowned for selecting a single input from multiple inputs & directing the binary information to output line?
a) Multiplexer						b) Data distributor
c) Both multiplexer and data distributor		d) DeMultiplexer

- The truth table for an S-R flip-flop has how many VALID entries?
a) 1		b) 2		c) 3		d) 4

- In which mode can we provide data to all the flip-flops simultaneously? 
a) loopback mode	b) Serial input mode	c) Parallel input mode       d) None of these

- Which among is a legal negative number

a) 4'd-3		b) 6'-d3		c) -6d'3		d) None of these

- Select the system task to suspend simulation.
a) $finish 		b) $monitor		c) $display		d) $stop

- In continuous assignment left hand side must be
a) Net		b) Reg		c) Scalar or Vector Net	d) Scalar or Vector reg

- In verilog what is the purpose of always block
a) To declare variables	b) To instantiate modules

c) To specify simulation time	d) To describe behavior using procedural statements

- Which operator is used for concatenation 
a) &		b) ()	c) [ ]		d) { }

PART – B

ANSWER ANY FIVE OF THE FOLLOWING                                                                     5X6=30

- a) Mention the worst case input and output voltages of standard TTL NAND gate.

b) With the help of circuit diagram explain CMOS NOT gate.                                     (2+4)

- a) Define De-multiplexer. Draw circuit diagram and output expression for 1x4 demux.

b) Define an encoder. Draw the circuit diagram and truth table of decimal to BCD

encoder.                                                                                                                  (3+3)

- a) With the help of a truth table explain NAND latch circuit.

b) Draw the circuit diagram of master slave J-K flip flop using gates.	               (3+3)

- a) Draw the circuit diagram and waveform for asynchronous decade counter.

b) Draw and explain Serial In Serial Out shift register.                           		  (3+3)

- a) Explain module with the help of example.

b) With an example each explain delays in dataflow modeling.			   (3+3)

- Explain blocking and non-blocking assignment statements with examples.

- a) Give any three differences between task and function.

b) With an example explain for statement.				                          (3+3)

PART – C

ANSWER ANY FIVE OF THE FOLLOWING                                                                      5x4=20

18. Find the fan out for given values.

- IOH  = 400µA         IIH  = 10µA     IOL  =  32mA     IIL  = 1.6mA

- IOH  =  1.25mA      IIH  =  20µA    IOL  =  3.2mA    IIL  =  0.4mA

19. Draw the truth table and circuit diagram of full subtrator using two half subtractor and one

OR gate.

20. Draw the circuit diagram of asynchronous mod-12 down counter.

21. Write design (gate level) and test module to implement a full adder.

22. Write design (dataflow) and test module to implement 4 to 1 Multiplexer.

23. Write design and test module to implement T flip-flop using behavioral modeling.

24. Write design (gate level) and test module to implement 4-bit comparator (only equality

condition).

*************************************************************************************************************