Register Number:

DATE: 12-4-19

ST. JOSEPH’S COLLEGE (AUTONOMOUS), BANGALORE-27

B.Sc. ELECTRONICS– II SEMESTER

SEMESTER EXAMINATION: APRIL 2019

EL 218 : Amplifiers and Oscillators

- Time- 2 ½ hrs		                                            		Max Marks-70

This paper contains three printed pages and three parts

PART-A

Answer any five questions.                                                                                        5X8=40

- Draw a neat circuit of two stage RC coupled amplifier and derive its voltage gain with the help of its equivalent circuit.

- Explain class B push pull amplifier circuit and derive its efficiency. What is meant by cross over distortion?

- a) Draw a Wein bridge oscillator circuit and explain its working.

b) Sketch and describe the block diagram of IC555.

(4+4)

- a) Explain the working of an astable multivibrator using transistors and derive its frequency of oscillation.

b) Draw the circuit for a voltage-controlled oscillator and give the expression for its output resonant frequency.

(6+2)

- Draw a dual input balanced output differential amplifier and arrive at its  differential voltage gain.

- a) Draw block diagram of operational amplifier and explain its working.

b) Write circuit diagram for op amp inverting  amplifier and derive its closed loop voltage gain.

(4+4)

- a) Write the circuit for an op-amp integrator and derive its output voltage. Draw the output waveform when the condition RC>>T is fulfilled.

b) With the help of a circuit explain the first order high pass filter and plot the frequency response curve.                                                                                                           (4+4)

PART-B

Answer any five questions:                                                                                        5X4=20

- Determine IB, IE, VB and VC for the Darlington transistor circuit given below.

- A power transistor working as class A amplifier is supplied from DC supply of 10V. If the maximum collector current is 50mA, find the power transferred to 4Ω speaker when connected to the transistor through a transformer directly. Determine efficiency?

- Hartley oscillator using split inductance of 2:1 ratio has an output of 450KHz with a capacitance of 20.9pf .Determine the corresponding inductance values.

- Determine the minimum and maximum frequencies for the given circuit.

- The 741 IC op-amp having following parameters is connected as a non-Inverting amplifier with R1=2kΩ, Rf=20kΩ, A=200000, Ri=1MΩ, Ro=75Ω and fo=10Hz. Calculate Af, Rif,R0f, Ff.

- Determine the output voltage for the given circuit. (All resistors shown are equal)

- Design a low pass filter for a mid-band gain of 9 and cut off frequency of 12 KHz.

PART- C

Answer any 5 questions:                                                                                   5x2=10

- Class C power amplifier is also known as tuned amplifier. Substantiate.

- Can we use 2 sections of RC in a Phase shift oscillator? Give reasons.

- Which multivibrator is called a one-bit memory device and why?

- LM317 IC regulator is better than 78XX series regulators. Explain.

- In a practical high pass filter gain rolls off at around 60KHz frequency. Give reasons.

- Second order Butterworth filters are advantageous over first order filters. Elucidate.

- Explain the significance of CMRR and what should be its ideal value?

------------------------------------------------------